FALCON’s Response to SAB Express RFQ
for the supply of SWEDI Sorter of Throughput 4000 PPH

May 15, 2023

Kind Attention –

Mr.  Turab Rahaman

SAB Express

Offer Ref: F22-00317; Date 09.05.2023

Subject – Revised Proposal for the supply of SWEDI sorter of throughput 4000 PPH.

Dear Sir,

We are pleased to submit our Revised Offer for Supply of SWEDI Sorter

As you will note, we have studied your requirements in great depth and, along with the information collected during the workshops and calls with you, we have put together a detailed technical proposal.

In subsequent sections, we have highlighted the capabilities and experiences of Falcon Autotech as well as the capabilities of the proposed software.

To conclude, I would like to add my personal commitment, on behalf of Falcon Autotech. As we move through the RFP process, please do not hesitate to contact me and my team. We will be pleased to assist you for any further information or clarifications that you might have.

tails

Contact Person		:         Sanyog Singh

Title			:         Assistant sales manager

Mobile			:         +91-8750052591

Email			:          Sanyog.singh@falconautoonline.com

Company Details

Company Name	              : 	Falcon Autotech Pvt. Ltd.

Regd. Office		:	57, Nimri Colony, Double Storey Flats, Delhi – 110052

Website		:	www.falconautotech.com

Linked ID		:	https://www.linkedin.com/company/falconautotech/mycompany/

Contents

1.	Glossary	5

2.	Company Profile	6

3.	Falcon’s Experience and Achievements in Sortation Space Globally	14

4.	Layout View	15

5.	Handled Parcel Spectrum	16

a.	Parcel size loadable on the sorter	16

b.	Parcels to be loaded on Sorter shall have the following characteristics:	16

c.	Parcel not loadable on the sorter	17

d.	Pictures of Sortable Shipments	17

6.	Key Modules	18

7.	Key Component Makes	26

8.	Proposed System Technical Details.	27

9.	Price Schedule	28

10.	Payment Terms.	28

11.	Warranty Period	28

12.	Delivery Schedule	28

13.	Exclusions	29

Response Quotation for Swivel Wheel Diverts

Proposal Reference - F22-00317.01

Falcon Autotech Private Limited

Plot No 308 & 309

Ecotech-1 Extension, Greater Noida

Uttar Pradesh- 201308

India.

Contact – Sanyog Singh

Assistant Manager

Mob - +91-8750052591

Sanyog.singh@falconautoonline.com

# Glossary

| S. No. | Term | Description |
| --- | --- | --- |
| 1 | RFQ | Request For Quote |
| 2 | PPH | Parcels Per Hour |
| 3 | ICR | Intelligent Character Recognition |
| 4 | PLC | Programmable Logic Controller |
| 5 | IT | Information Technology |
| 6 | BOQ | Bill Of Quantity |
| 7 | I/O | Input/ Output |
| 8 | PC | Personal Computer |
| 9 | UPS | Uninterrupted Power Supply |

# Company Profile

# Falcon’s Experience and Achievements in Sortation Space Globally

- Ranked among Top 20 Sortation System Suppliers globally. Only company from APAC/EMEA Region.

- Currently possess one of the World’s largest portfolio in Sortation Technologies: 7 In-house technologies.

- Total installed capacity of 6 Million Parcels per day worldwide.

- Only company to be able to offer a Fully Integrated AMS.

Kind Attention –

# Layout View

Process notes

- Swivel wheel diverter is to be installed at ground floor and the basement.

- The system has 2 infeed points.

- At First Infeed Point:

- Shipments are loaded on to the infeed conveyor in a singulated manner.

- It will pass through the X-ray machine.

- The operator will check the shipments, reject the wrong ones, and orientate the rest in a manner that the bar code is on the top.

- At Second Infeed Point:

  - The pallets are fed into the system.

  - It will pass through the X-ray machine.

  - The operator will bring down the pallet from the conveyor, With the help of pallet handler. break it and fed it to the sorter in a singulater manner ensuring barcode facing top.

  - If the X-ray machine of first infeed will not work, then the shipments will also come from the second (pallet) x-ray machine. These shipments are being transferred from x-ray machine to the feedline through powered flexible roller conveyor.

  - Also, whenever the throughput number high pallet x- ray feedline used for shipment feeding.

- These shipments are then scanned by automatic scanner.

- The first sorting point is at ground floor. Rest of the shipments will be moved to the basement with the help of spiral chute.

- In the basement, there will be an operator who will orientate the shipment and ensure that the barcode is at the top.

- Then these shipments then further scanned by automatic barcode scanner.

- Based on 5 locations, the shipments will be sorted.

- There is rejection chute as well for rejected shipments.

# Handled Parcel Spectrum

Dimensions Minimum Maximum

## Parcel size loadable on the sorter

| Product Range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Transported Items | Size | Length (mm) | Width (mm) | Height (mm) | Weight (Kg) |
| Carton boxes, Poly Bags | Maximum | 900 | 500 | 500 | 35 |
|  | Minimum | 150 | 150 | 50 | 0.5 |

## Parcels to be loaded on Sorter shall have the following characteristics:

- Centre of Gravity of item must not move during conveyance or sorting.

- Item must not have magnetic content, otherwise behavior of parcel cannot be guaranteed.

- Liquid or fragile material, to avoid breaking, spillage or leakage, such as wine bottles, metal cans of paint are designated as non- conveyable items.

- Parcels shall be perfectly and safely packaged: protrusion or open surfaces are not allowed.

- Plastic ropes shall be perfectly adherent to the surface of the package.

- Loose parts on loads and load carriers, such as adhesive tape, stickers, slips of paper, straps, wrap foil etc. are designated as non- conveyable items.

- All items with the risk of being damaged during the transport on an automatic sorting system or

damaging the sorting system.  They must be robust enough to avoid disintegration of container material and loose of contents in the sorting process.

- Item packaging shall have enough grip to be handled on the belts during the acceleration and

referencing phases.

- Items shall not have slippery surfaces and must be able to withstand acceleration of the items on the belt during the start-stop phases (accelerations up to 0.5 g shall be assured without any sliding or tumbling of the items on the belt conveyor).

- The parcels must have at least one flat and regular surface providing enough stability during conveyance.

- Unstable items with a risk to roll or tumble on the sorting system, such as spherical or cylindrical items

- All shapes are permitted except spherical, cylindrical, or alike unstable items & shapes.

- All usual packaging materials are permitted (including paper, carton, plastics, plastic foil, rope, tape, textile, and wood)

## Parcel not loadable on the sorter

All products that are not within the range as described here, are considered non- conveyable products, and must be taken out of the main sorter flow by the operators.

- Items that have a spherical or cylindrical shape.

- Items that are packed in material that can damage the conveyors or the sorter.

- Items that have sharp points (e.g. Nails) or sharp edges, that can damage the conveyors or the sorter.

- Fragile parcels with contents not sufficiently secured

- Items that have been classified as dangerous are designated.

- Wet items are designated.

- Items with anti-slip treatment.

- Items with protruding parts.

- Items with sharp edges.

- Inadequately packed items that could be damaged during automatic transportation.

- Electrostatically loaded items.

## Pictures of Sortable Shipments

# Key Modules

  - Automatic Feedlines

Fully automatic feedline is used in the system to induct parcels directly on to the sorter

Feedlines consists of below components.

    - Infeed conveyors.

Shipments are loaded on this conveyor in a singulated manner after that it will pass through X- ray machine.

    - Plastic Modular Belt Conveyor

Falcon’s Belt Conveyors are modular & robust in design. MS profile is used to build conveyor frame.

Some Salient Features of Falcon’s Modular Belt conveyor:

- Low Noise

- Maximum uptime.

- Minimal maintenance

- High safety standards.

- Fastest ROI.

| Specification | UOM | Remark |
| --- | --- | --- |
| Manufacturer Name | Name | Falcon Autotech |
| Plastic Modular Belt Material | Type | POM with PBT Pins, Pitch-25.4 mm, Pin Dia- 5mm |
| Plastic modular Belt Make/Model No. | Make | Movex/XP254 CR |
| Sprocket Size & No. of Sprockets on Drive End & Tail End | Type | 5/146.4 mm |
| Sprocket material | MOC | POM |
| Chassis | mm | MS, 3mm Thick, Powder Coated-75 Microns |
| Wear Strip Material | MOC | UHMW PE1000 |
| Wear Strip Pitch | mm | 150 (Strip Width - 40mm) |
| Return Roller type | Type | DIA- 50 mm/20303 |
| Return Roller Pitch | mm | 800 |
| Driven Shaft | Type | MOC-EN9 Solid Shaft hardened to BHN 240, Size-60SQ MM, Zinc Plated -25 microns, Bearings-UCF-210D1 |
| Duplex Sprocket | MOC | MS |
| Duplex Chain | MOC | MS |
| Drive Shaft | MOC | EN 24 |
| Conveyor Under guarding | Type | GI SHEET (1MM THICK) |
| Type of Guides | Type | Steel guides at 50mm/400mm height basis the position of conveyor on both sides from Conveyor Belt Surface. |

    - Shipment Loading Conveyor

It’s a approx. 2.5 m long. Powered Belt Conveyor for Shipment Loading. It is running at approx. 800 mm from ground Level.

    - Detachable flexible conveyor.

Flexible conveyors are used to load and unload the shipments from truck. It let you change discharge points by rolling and curving conveyor sections to fit your needs. They're commonly used at shipping docks or the discharge end of a conveyor system to connect to truck trailers. A single conveyor can serve multiple dock doors.

| S. No | Description of Goods | Details |
| --- | --- | --- |
| 1 | Conveyor Width Between Frame | 800mm |
| 2 | Conveyor Open Length | 10000mm |
| 3 | Conveyor Closed Length | 5000mm |
| 4 | Roller Close Pitch | 60mm |
| 5 | Roller Open Pitch | 120mm |
| 6 | Adjustable Height | 950 - 1200mm |

    - Orientation Conveyor.

It’s a approx. 6 m long. Powered Belt Conveyor for Shipment Orientation running at approx. 800 mm from ground Level.

    - Spacing Conveyor

It’s a Special Conveyor, used to create space between parcels

    - Buffer conveyor

High-speed conveyor, used to create appropriate space between parcels/boxes and synchronize the infeed into the Sorter.

    - Intelligent merge / Angle merge.

This is 30° triangular Dual Belt high-speed conveyor used for inducting parcel/boxes directly on to the sorter. The Belts are Strip Belts for smooth parcel movement.

    - Spiral chute.

The Spiral chute is a compact and versatile elevating solution. It is specially designed chute which will take the shipments from G0 to Basement.

  - Barcode Scanning

    - Automatic Barcode Scanning

System is Integrated with Automated Line Scan Camera Technology (Sick ICR) for scanning 1D & 2D Barcodes.

| Specification | UOM | Remark |
| --- | --- | --- |
| Manufacturer Name | Name | SICK- ICR 890 |
| Barcode Type | Type | 1D & 2D |
| Module Size | Mils | X or higher |
| Shipment Size | Mm | Min: 150 x 150 x 5 Max: 900 x500 x 500 |
| Number of Codes | Nos | 1 |
| Location | Side | Top |
| Orientation | Type | Top side |
| Color of bars | Color | Black |
| VMS | Nos | 1 |
| Under Foil | yes/no | No |

    - Handheld barcode Scanner

System is equipped with handheld barcode scanners capable for scanning 1D barcodes for Semi Auto mode.

  - Sorter: Swivel Wheel Divert

Swivel Wheel Diverter (SWEDI) is a non-impact sortation module for linear sortation lines, ideal for medium volume sort centres. SWEDI's modular design allows it to save space while also allowing for future expansion of sorting point

Technical specifications of SWEDI Unit

| Items | Make |
| --- | --- |
| SWEDI Unit’s Dimensions | 815 mm (L) x 915 mm (W) |
| Load Capacity | 35 KG |
| Designed Throughput | 4000 PPH |
| Conveying Drive Unit | 48V, 50 Watt DC powered smart roller |
| Diverting Drive Unit | Servo Motor; 220V, 50HZ, 750W, Single Phase |
| Conveying Speed | Up to 120 m/min |
| Diverting Angle | 90 Degrees at double sides |
| Frame Material | MS Powder Coated Steel Frame |
| Stand Support | MS Powder Coated Steel Frame |
| PLC | Siemens |
| Servo Motor | Delta |
| Noise Level | <75 Db when measured from 1 m in a silent warehouse. |

  - Sliding Chutes

Parcels are collected in the in trolley with the help of sliding chute. The chutes are equipped with Chute full sensors and tower lamps.

  - Steel works.

Platform 3 SQM (Approximately).

# Key Component Makes

| S. No. | Description | Makes |
| --- | --- | --- |
| 1 | PLC | Siemen's |
| 2 | HMI | Siemen's |
| 3 | Control Cabinet & Accessories | BCH / Rittal |
| 4 | Drives | Allen Bradely/ Lenze/ Siemen's |
| 5 | Drive Motor and Gearbox | SEW/ Bonfigloili |
| 6 | Sensors | Sick, P&F |
| 7 | Bearings | SKF, NBC, NTN |
| 8 | PVC Belt | Forbo, Derko, |
| 9 | Rollers Make | Pulse/ Falcon |
| 10 | Automatic Barcode Scanners | SICK/ Cognex |
| 11 | Handheld Scanners | Motorola/ Honeywell |

# Proposed System Technical Details.

| Mechanics |  |  |  |
| --- | --- | --- | --- |
| Pos. | Qty. | Description | Value |
| FM1 | 1 | Conveyor Automation |  |
|  |  | 02 x Angle merge. 13 x Powered Belt Conveyor; L-85 m 02 x Scanning Conveyor 03 x Spacing Conveyor 03 x Buffer Conveyor 03 x Receiving Conveyor 01 x 60-degree turn 01 x Detachable flexible conveyor L- 10m 01 x Modular Conveyor; L- 5.8 m; Width 1200 mm 02 x Gravity Roller Conveyor; L-4.8 m | Width-1000 mm |
| FM2 | 1 | Falcon Sorter-Swivel Wheel Diverter |  |
|  |  | System Width | 915 mm |
|  |  | System Length | 815 mm |
|  |  | 1 x Automatic Barcode Scanning System. |  |
|  |  | 07 X Swivel Wheel Divert Units. 06 X Collection chute. 01 X Spiral chute 01 X Rejection |  |
| Controls |  |  |  |
| FC1 | 1 | Controls, IT and Wiring |  |
|  |  | Software Package |  |
|  |  | Control Panels |  |
|  |  | Sorter Control | Included |
|  |  | Electrical & Control Hardware |  |
|  |  | Switches |  |
|  |  | Drive panels |  |
|  |  | Operator Panels |  |
|  |  | Internal & Field Wiring |  |

# Price Schedule

Below chapter consists of Price of Falcon’s Mechanics and Controls.

| S. No. | Description | Price (USD) |
| --- | --- | --- |
| 1 | Feedline and Sorter | $ 287,869 |
| 2 | Conveyors | $ 168,159 |
| 3 | Chutes | $ 5,901 |
| 4 | Falcon WCS and Integration | $ 8,684 |
| 5 | Tool Kit | $ 1,155 |
| 6 | Project Management and Engineering Charges | $ 12,385 |
| 7 | Packaging & forwarding | $ 23,178 |
| 8 | Operator Platform | $ 470 |
| Grand Total (USD) Excluding Taxes. |  | $ 507,802 |

Note- Above price includes reversible feature in some conveyor modules; Details will be shared in DAP Stage. IT does not includes scanning the reversible direction.

Optional Price for Volume Measurement System is USD 26,600.00

Total Price is to be understood-

- Including Packing & Forwarding Charges

- Excluding Installation & Commissioning Charges

- Freight- Excluded

- Taxes Extra as applicable

- Unloading in Client’s Scope

- Laydown Area to be provided by SAB Express.

# Payment Terms.

The Supplier Quotation is based on the following payment schedule:

| Payment %age | Project Stage |
| --- | --- |
| 50 % | Advance. |
| 50 % | After inspection and clearance before dispatch. |

# Warranty Period

Falcons offered System comes with a standard warranty of 1 year and post completion of warranty, customer can opt for Extended warranty.

# Delivery Schedule

22 to 24 weeks Ex-Works, Greater Noida from the date of PO, full payment, and DAP.

# Exclusions

The scope of supply includes all parts which are defined in the Supplier’s quotation.

All other parts which are not defined in the Supplier’s quotation do not belong to the Supplier’s scope of supply and are excluded. The following parts are also excluded:

- Server PC & Server Rack

- Excluding installation and Commissioning

- Construction Power

- Steel Works- If not specified

- Collection Bin

- Building infrastructure; building structure, doors, fire exits, levelling devices, building extinguisher and fire alarm system, building heating and lighting system.

- Electrical power supply and wiring to the main control cabinets

- UPS for Controls and Drives

- Electrical Power for Installation

- Network Cabling up to the Main Server Rack

- Intermediate wiring to parts which are to be supplied by the Purchaser/others

- Emergency/Uninterruptable power supply

- Fire-alarm and fire protection devices

- Traffic and route markings

- Laydown Area

- Ram protection devices

- Cat walks, bridges, maintenance aisles and platforms

- Assembly tools like forklifts and hoisting machines

- All kind of network incl. Local Area Network (LAN/WLAN), exceeding the scope described in Scope of Supply

- Any Kind of Civil work

- Any adjustment of the Supplier’s scope of supply to local rules and regulations

- X-Ray machines

- Roller cages/ Pallets

- Simulation and 3D animation of the sorter system

- Interface with other equipment not specified in this offer.

- Provision of facilities for the control room (furniture, air conditioning, heating, etc.).

- The supply and installation of fencing around the different corridors.

- Any item specifically indicated as not forming part of the subject matter of the Seller's supply in the offer documentation.